Embedded Controller programing for Real Time Systems Final Exam

Name: Hsuankai Chang

Email: hsuankac@umich.edu

- Create a software interrupt and use one of the non-used IRQ.

  - You will write your own IRQ Handler, callback method and will enable interrupt.

  - You could use any unused IRQ but let’s use 48 for this assignment.

  - Create another menu with character ‘s’, which will generate the software interrupt and you should print the message “SW Interrupt detected”.

Answer:

- Create the user menu for the final exam first. Noted that I disable the IWDG and RTC init for 1 – 4.

- In MX_GPIO_Init, enable the interrupt for FMC_IRQn and set its priority.

- Create the switch case for user input character.

- Implement the FMC_IRQHandler in stm32l4xx_it.c file. In the IRQ handler, I just set the interrupt flag to 1.

- Build and test the program, test is successful.

- Create a method myDelay1() using timer2, which will take input in Millisecond.

  - Replace the existing HAL_Delay() with myDelay1() for Blue LED.

  - Program timer 2 registers to implement the delay.

Answer:

- First, specify the clock for TIM2. I decided to use HSI as a clock source.

- Second, defined the TIM2 parameters. 16000000 / 16000 = 1000, which is 1ms per counter.

- Setup the NVIC settings.

- Implement the Delay1 function.

- Implement the logic in switch case.

- Build and test the code, test is successful.

- Create a method myDelay2() using SysTick, which will take input in Millisecond.

  - Replace the existing HAL_Delay() with myDelay2() for Green LED.

  - Program SysTick registers to implement the delay.

Answer:

- From SystemClock_Config function -> HAL_InitTick function -> HAL_SYSTICK_Config function -> SysTick_Config function, we have configured the systick register for 1 ms

- We can utilize the systick initialization function to code the delay 2 function.

- Code the switch case for green LED.

- Build and test the code, test is successful.

- Use Timer 3 to count events.

  - Program the timer3 in MX_TIM3_Init() with values , which will expire every second.

  - Create a menu with character ‘t’ to start the timer3.

  - You will need to implement HAL_TIM_PeriodElapsedCallback() method to know when 1 sec has lapsed.

  - Count for 10 (equivalent to 10 sec) in main loop and stop the timer3 when count reaches 10.

  - Print the log message - "Total counted timer3 event = %d\r\n"

Answer:

- Set up the TIM3 parameter. 16000000 / 16000 / 1000 = 1(sec).

- Enable the interrupt for TIM3.

- Coded up the switch case in menu.

- Coded up the IRQ call back function.

- Build and run the code, test is successful.

- Uncomment MX_IWDG_Init() code to test the watchdog.

  - Program the prescaler, window and reload value for timeout of 0.5 sec.

  - Pet the watchdog in main () code – to avoid board reset.

  - There should be no reset and software should be working properly.

  - Now simulate a failure by introducing a delay more than 0.5 sec to miss the watchdog pet.

  - Create a menu with character ‘w’ and delay of 1 sec. Since 1 sec is more that 0.5 sec, SW will miss the pet and unit will reset.

Answer:

- Configure the IWDG parameter. (1 / (32000 / 32)) * 500 = 0.5 (sec).

- Pet the watchdog timer, comment out the delay, no reset occurs.

- Coded switch case for w character.

- Build and test the code, test is successful, processer reset if delay more than 0.5 sec.

- Uncomment MX_RTC_Init() to test RTC Alarm.

  - Set the alarm for hour 0, minute 1.

  - Use RTC callback to detect the alarm.

Answer:

- Set up alarm A in RTC. 1 minutes timeout.

- Enable the interrupt line.

- Implement the call back function.

- Build and run the code, test is successful.

- Notes:

  - Since I use UART1 in interrupt mode, I set the UART1 interrupt to highest priority so I can call logMsg function inside other IRQ handler or call back functions.